Two-Layer Infiltration

via LID Control Reuse

One New Parameter. Zero New Sections. Full Two-Layer Soil Modeling.

Robert E. Dickinson

Autodesk Water Technologist  •  Chair, SWMM5+ TAC

February 26, 2026  •  SWMM5.2+ / SWMM5+

# 1. Design Philosophy

The LID soil layer already defines thickness, porosity, field capacity, wilting point, Ksat, suction, and initial moisture deficit — with all parsing, validation, and unit conversion already working in the SWMM5 codebase.

Instead of creating a new infiltration method with 14 parameters and a new INP format, this approach makes three minimal changes:

- Add one new keyword to [LID_CONTROLS]: SOIL2 (identical format to SOIL) for the lower layer

- Add one new parameter to [LID_USAGE]: UseAsInfil (0 or 1) at the end of the line

- When UseAsInfil = 1, the subcatchment’s pervious area infiltration uses the LID’s SOIL (upper) and SOIL2 (lower) layers instead of the default [INFILTRATION] method

Comparison: Old vs. LID Reuse Approach

| Aspect | Old (14-param) Approach | LID Reuse Approach |
| --- | --- | --- |
| New INP parameters | 14 per subcatchment | 1 (UseAsInfil flag) |
| New INP section format | Yes | No — existing LID format |
| New [OPTIONS] keyword | Yes — TWOLAYER | No |
| New parsing code | ~200 lines | ~20 lines |
| Reuses LID validation | No | Yes |
| Backward compatible | New INP format | 100% |
| Per-subcatch control | All or nothing | Per-subcatchment |
| Soil profile reuse | Must duplicate | Shared via LID controls |

Multiple subcatchments can reference the same LID control, sharing the same soil profile definition. Define it once, use it everywhere.

# 2. Source Code Changes (Minimal)

## 2.1 Changes to lid.h — Add SOIL2 to TLidProc

Add a second TSoilLayer member and a flag to the existing TLidProc structure:

typedef struct {

// ... existing members ...

TSoilLayer  soil;       // Existing (SOIL line)

TSoilLayer  soil2;      // NEW: lower layer (SOIL2)

int         hasSoil2;   // NEW: 1 if SOIL2 defined

}  TLidProc;

The TSoilLayer structure already has thickness, porosity, fieldCap, wiltPoint, kSat, and suction. No changes needed to it.

## 2.2 Changes to lid.c — Parse SOIL2 + UseAsInfil

In lid_readProcParams(), add a SOIL2 case using identical parsing logic as SOIL:

else if (match(tok[0], "SOIL2")) {

lidProc->soil2.thickness = x[0] / UCF(RAINDEPTH);

// ... same as SOIL parsing ...

lidProc->hasSoil2 = 1;

}

In lid_readGroupParams(), read UseAsInfil at the end of [LID_USAGE]:

if (useAsInfil == 1) {

lidTwoLayer_activate(j, lidIndex);

}

## 2.3 Changes to subcatch.c — Override Infiltration

if (lidTwoLayer_isActive(j))

infil = lidTwoLayer_getInfil(j, tstep, rainfall, runon, depth);

else

infil = infil_getInfil(j, tstep, rainfall, runon, depth);

## 2.4 Summary of All Changes

| File | Change | Lines | New? |
| --- | --- | --- | --- |
| lid.h | Add soil2 + hasSoil2 to TLidProc | ~3 | No |
| lid.c | Parse SOIL2 keyword (copy of SOIL) | ~15 | No |
| lid.c | Parse UseAsInfil in [LID_USAGE] | ~12 | No |
| subcatch.c | if/else for lidTwoLayer_getInfil | ~5 | No |
| project.c | Init and close calls | ~4 | No |
| lid_twolayer.h | Header file | ~60 | Yes |
| lid_twolayer.c | Implementation | ~250 | Yes |

Total existing code touched: approximately 39 lines. Everything else is in the two new files.

# 3. INP File Format Changes

## 3.1 [OPTIONS] — No Change

The existing INFILTRATION keyword is unchanged. The standard method remains the default for subcatchments without UseAsInfil = 1.

## 3.2 [LID_CONTROLS] — New SOIL2 Keyword

[LID_CONTROLS]

SandyLoamClay    BC

;;Name           Layer  Thick  Por   FC    WP    Ksat  Suct  IMD

SandyLoamClay    SOIL   12.0   0.453 0.190 0.085 0.43  4.33  0.30

SandyLoamClay    SOIL2  24.0   0.464 0.310 0.187 0.04  8.22  0.15

SOIL2 uses the identical 7 parameters as SOIL. SOIL = upper layer, SOIL2 = lower layer.

## 3.3 [LID_USAGE] — New UseAsInfil Column

[LID_USAGE]

;;Subcatch  LID            Num Area Wid  Sat  FrI  ToP  Rpt  Drn  FrP  UseAsInfil

S1          SandyLoamClay  0   0    0    0    0    0    *    *    0    1

UseAsInfil = 0 (default): Normal LID behavior. UseAsInfil = 1: SOIL+SOIL2 replace the subcatchment’s default infiltration.

## 3.4 Backward Compatibility

- SOIL2 is optional — LID controls without it work exactly as before

- UseAsInfil is optional — defaults to 0 if omitted

- No changes to [OPTIONS], [INFILTRATION], or any other section

- Existing INP files produce identical results

# 4. Theory — The LID Percolation Equation

The core equation comes from lid_proc.c :: soilFluxRates():

percRate = kSat * exp(-15.0 * (1.0 - theta / porosity));

This approximates Brooks-Corey unsaturated hydraulic conductivity:

| Saturation | θ/n | Percolation Rate | % of Ksat |
| --- | --- | --- | --- |
| Saturated | 1.0 | Ksat × 1.000 | 100% |
| 90% | 0.9 | Ksat × 0.223 | 22.3% |
| 80% | 0.8 | Ksat × 0.050 | 5.0% |
| 70% | 0.7 | Ksat × 0.011 | 1.1% |
| 50% | 0.5 | Ksat × 0.00055 | 0.055% |

The steep exponential decay realistically captures how water moves through unsaturated soil. Applied twice: upper → lower and lower → deep.

Moisture Balance Equations

Upper:  dθ_u/dt = (infil - perc_upper - ET_u) / thickness_upper

Lower:  dθ_l/dt = (perc_upper - perc_lower - ET_l) / thickness_lower

Where infil = Green-Ampt from surface, perc = LID equation, ET = evapotranspiration.

# 5. Typical Soil Parameter Values

Reference values for USDA texture classes (for both SOIL and SOIL2 lines):

| Soil Type | Porosity | Field Cap | Wilt Pt | Ksat (in/hr) | Suction (in) |
| --- | --- | --- | --- | --- | --- |
| Sand | 0.437 | 0.062 | 0.024 | 4.74 | 1.93 |
| Sandy Loam | 0.453 | 0.190 | 0.085 | 0.43 | 4.33 |
| Loam | 0.463 | 0.232 | 0.116 | 0.13 | 3.50 |
| Silt Loam | 0.501 | 0.284 | 0.135 | 0.26 | 6.69 |
| Clay Loam | 0.464 | 0.310 | 0.187 | 0.04 | 8.22 |
| Clay | 0.475 | 0.378 | 0.265 | 0.01 | 12.45 |

# 6. Documentation Updates Required

## 6.1 SWMM5 User’s Manual

- Chapter on LID Controls: Add SOIL2 keyword description (same 7 parameters as SOIL, defines lower soil layer)

- Chapter on LID Usage: Add UseAsInfil parameter description

- Add note that SOIL + SOIL2 with UseAsInfil = 1 provides two-layer infiltration for the pervious area

## 6.2 SWMM5 Technical Reference

- Add section on two-layer infiltration: Green-Ampt upper layer + LID percolation equation between layers

- Document moisture balance equations for both layers

- Show percolation rate table at various saturation levels

## 6.3 Appendix D — Input File Format

- Add SOIL2 to list of [LID_CONTROLS] keywords with parameter descriptions

- Add UseAsInfil to [LID_USAGE] parameter list

# 7. Testing Recommendations

- Backward compatibility: Existing INPs with no SOIL2 or UseAsInfil must produce identical results

- Mass balance: Rainfall = runoff + Δstorage(upper) + Δstorage(lower) + deep perc + ET

- Consistency: With identical SOIL and SOIL2 properties, compare with single-layer Green-Ampt

- Independent state: Multiple subcatchments sharing one LID control must track moisture independently

- Mixed models: Subcatchments with and without UseAsInfil in the same run

# 8. Future Enhancements

- Groundwater coupling: deep percolation → SWMM5 groundwater recharge

- SOIL3, SOIL4 for N-layer profiles

- Temperature-dependent Ksat for frozen ground

- Capillary rise from lower to upper layer

- Layer-specific root zone evapotranspiration